TASK 2 SUBMISSION

Design & Integrate Your First Memory-Mapped IP

Simple GPIO Output IP (Write-Only)

# 1. GPIO IP RTL File

File submitted: gpio_ip.v

The GPIO IP is implemented as a standalone Verilog module (gpio_ip.v) with the following specification:

- One 32-bit register (gpio_reg)

- Writing to the register updates the gpio_out output signal

- Reading the register returns the last written value (gpio_rdata)

- Fully synchronous design — register updates on rising clock edge

- Active-low reset — register clears to 0x00000000 on reset

Module interface:

module gpio_ip (

input  wire        clk,          // system clock

input  wire        resetn,        // active-low reset

input  wire [29:0] mem_wordaddr,  // word address from CPU

input  wire        isIO,          // IO page select (mem_addr[22])

input  wire        mem_wstrb,     // write strobe (|mem_wmask)

input  wire [31:0] mem_wdata,     // write data from CPU

output wire [31:0] gpio_rdata,    // readback to IO_rdata mux

output wire [31:0] gpio_out       // GPIO output port

);

# 2. SoC Integration

File modified: riscv.v  |  Files unchanged: emitter_uart.v, clockworks.v

Six targeted changes were made to riscv.v to integrate the GPIO IP. No changes were made to the Memory module, Processor module, or any other peripheral.

| Change # | What Was Changed | Location in riscv.v |
| --- | --- | --- |
| 1 | `include "gpio_ip.v" | Top of file, after emitter_uart include |
| 2 | output [31:0] GPIO_OUT port | module SOC port list |
| 3 | localparam IO_GPIO_bit = 3 | IO localparam block, after IO_UART_CNTL_bit |
| 4 | wire [31:0] gpio_rdata declaration | Wire declarations inside SOC |
| 5 | gpio_ip GPIO(...) instantiation | After UART instance |
| 6 | gpio_rdata added to IO_rdata mux | wire [31:0] IO_rdata = ... expression |

Integration diff — key code change (IO_rdata mux):

// BEFORE:

wire [31:0] IO_rdata =

mem_wordaddr[IO_UART_CNTL_bit] ? { 22'b0, !uart_ready, 9'b0}

: 32'b0;

// AFTER:

wire [31:0] IO_rdata =

mem_wordaddr[IO_UART_CNTL_bit] ? { 22'b0, !uart_ready, 9'b0} :

mem_wordaddr[IO_GPIO_bit]      ? gpio_rdata                   :

32'b0;

# 3. Simulation Log & Waveform

## 3.1 Terminal Output (iverilog + vvp)

All 10 tests passed with 0 failures. exit code=0 confirms clean simulation completion.

## 3.2 GTKWave Waveform

The waveform clearly shows the full write-then-readback cycle for each test vector. Key observations:

- resetn goes HIGH → gpio_reg immediately shows 0x00000000

- isIO and gpio_sel_w pulse HIGH together for every write

- gpio_reg latches each new value on the clock edge following gpio_sel_w

- gpio_rdata and gpio_out track gpio_reg identically at all times

- BADOBAD0 appears in mem_wdata (Test 9) but gpio_reg stays at 12345678 — address decode correctly blocks the write

# 4. Short Explanation

## 4.1 Address Used

The GPIO register is mapped at address 0x00400008 in the memory map.

Address derivation:

- mem_addr[22] = 1  →  IO page selected  →  base address 0x00400000

- IO_GPIO_bit = 3   →  1-hot word-address bit 3

- Word address 0x00000008 has bit 3 set  →  byte offset = 0x08

- Final address: 0x00400000 + 0x08 = 0x00400008

IO Address Map (complete):

IO_LEDS_bit      = 0  →  0x00400000  (write only)

IO_UART_DAT_bit  = 1  →  0x00400004  (write only)

IO_UART_CNTL_bit = 2  →  0x00400008  (read only)

IO_GPIO_bit      = 3  →  0x00400010  (read/write)

## 4.2 How the CPU Accesses the IP

The CPU uses standard RISC-V SW (store word) and LW (load word) instructions to access the GPIO register. In C firmware:

#define GPIO_REG  (*(volatile unsigned int*)0x00400010)

GPIO_REG = 0xDEADBEEF;         // SW instruction → writes to GPIO

unsigned int val = GPIO_REG;   // LW instruction → reads back value

Write path:

CPU executes SW → mem_wmask goes non-zero → mem_wstrb (= |mem_wmask) goes HIGH → isIO & mem_wstrb & mem_wordaddr[3] asserts gpio_sel_w → gpio_reg latches mem_wdata on the next rising clock edge → gpio_out immediately reflects the new value.

Read path:

CPU executes LW → mem_rstrb goes HIGH → gpio_rdata (combinational) returns gpio_reg when mem_wordaddr[IO_GPIO_bit] is high → feeds into the IO_rdata mux → returned to CPU as mem_rdata.

## 4.3 What Was Validated in Simulation

| Test | Value Written | What It Verifies | Result |
| --- | --- | --- | --- |
| Reset | — | Register initializes to 0x00000000 on reset | PASS |
| Test 1 | 0x00000000 | All-zero write and readback | PASS |
| Test 2 | 0xFFFFFFFF | All-ones — no bits stuck at 0 | PASS |
| Test 3 | 0x00000001 | LSB (bit 0) independently writable | PASS |
| Test 4 | 0x80000000 | MSB (bit 31) independently writable | PASS |
| Test 5 | 0xAAAAAAAA | Alternating pattern — no adjacent bit coupling | PASS |
| Test 6 | 0x55555555 | Inverse alternating pattern | PASS |
| Test 7 | 0xDEADBEEF | Arbitrary value — general correctness | PASS |
| Test 8 | 0x12345678 | Overwrite — register updates correctly | PASS |
| Test 9 | 0xBAD0BAD0 (blocked) | Write ignored when isIO=0 — address decode OK | PASS |

Summary:

- 10 / 10 tests passed — 0 failures

- gpio_out and gpio_rdata matched the written value in every test

- Waveform confirmed synchronous latching on clock edge

- Waveform confirmed combinational readback with zero additional latency

- Address decode logic verified — writes with isIO=0 are correctly ignored